# The Baja Caja (Baja Box) Urine-Diverting Composting Toilet

In 2000, my friends who own a retreat lodge in Baja asked me to design a composting toilet that was attractive enough to be used by guests but could be moved around. David Del Porto developed a system contained in a wooden box. We used a CITA Mexican Urine-Diverting Toilet (available from Ecovita), but it can just as easily be used with a Separett Privy Kit (also available from Ecovita; www.ecovita.net). You can even make your own urine diverter with a funnel, but once you’ve used the easy, attractive and ergonomically correct Privy Kit, you won’t go back!

Alas, after a week of figuring out the measurements then making one of these, I lost the original plans. Recently, I asked flamenco guitarmaker Stephen Faulk to create plans. Here they are. This one uses a 5-gallon bucket inside; I’ve used a 40-gallon plastic bin with aeration holes covered with insect netting. But this will work. Stephen forgot to send photos of the urine tubing and vent stack. More photos will be added when I return from Baja in February 2005. – Carol Steinfeld at Ecovita

# Overall Dimensions

Width 31"

Depth 24"

Height 18 1/2"

Sides:  18" x 24 "

Front and back panel 18" x 30 "

Seat side of top panel 20" x 24"

Vent side of top panel 11" x 24"

Vent Hole 4 1/8" dia.

Seat hole template provided with Separett box.

# Materials

18 linear ft. 2"x 2"  Doug Fir

One 4' x 8' sheet 1/2" or 5/8" plywood

AND one half sheet of same.

( note: one sheet will not make entire box. )

1 1/4" drywall screws

Carpenters wood glue.

Basic power tools,

Screw gun, jig saw and or Skil saw

Assembly:

Cut side and back panels from plywood. Attach 2" x 2" lumber around perimeter of front and back panels as shown in pictures.

Screw sides to front and back to make box structure.

Cut top panel to fit entire box top.

Measure 19" from inside of box side panel on front and back and attach a 2" x 2" strut across the top of the  box at this point. This will provide a ledge for the vent panel top   fasten to.

Cut the top panel into two parts making sure when the panels are set on the top of the box the 2 x 2 supports the edge of each panel.

The top panels will measure approx.:

Vent side: 11" x 24"

Seat side 20 " x 24"

Holes for seat and vent:

Mark a 4 1/8" diameter hole on the vent panel. The center of the hole will be located a third of the way back on the panel from the front edge of the box and in the center of the 11" dim.  See photos.

Use template provided on Privy Kit box to trace  pattern for seat. Place pattern so that front edge of foam seat is 3 to 3 1/2" from front edge of box. Cut hole with jig saw. See photos.

Fit dark blue separator element on hole in seat side of top panel. Attach with screws.

Attach vent panel to top over 2 x 2 strut.

Fit seat side of top next to vent panel. Attach scrap wood registration bars under seat to keep it from sliding on top. Scraps of 1" x 1" material can be used of hinges can be attached to back of seat panel. Note : hinges may make it more difficult to access waste holding bucket.

Fit blue foam seat cover over Privy Kit.

5 gallon bucket will fit under separator leaving room for vent attachment and urine hose to be attached to front bowl of diverter.